# PCSM ASSIGNMENT 1

# 1. Process of Creating an Emergency Boot Disk

An emergency boot disk is a removable storage device, such as a USB flash drive or CD/DVD, that contains essential system files required to start a computer when the main operating system fails to boot. It is used for system recovery, troubleshooting, and repairing startup problems. The process of creating an emergency boot disk generally involves the following steps:

Step 1: Prepare the Necessary Storage Device
Select a suitable storage medium, normally a USB drive with at least 1GB–4GB of space. Ensure that the device is empty or back up any important files, as the process may erase its contents.

Step 2: Access the Boot Disk Creation Tool
Most modern operating systems provide built-in tools for creating a system recovery or boot disk. Examples include:
• Windows: “Create a Recovery Drive” or “Windows Media Creation Tool”
• Linux: Startup Disk Creator or Rescue USB tools

Step 3: Select the Drive and System Files
Insert the USB drive and select it within the tool. On Windows, you may choose the option “Back up system files to the recovery drive” to include troubleshooting utilities that help repair system errors.

Step 4: Create the Boot Disk
Start the process and allow the system to copy essential boot files onto the USB drive such as boot manager files, recovery utilities, and command-line tools.

Step 5: Test the Boot Disk
Restart the computer and press the appropriate key to access the boot menu. Select the USB drive and confirm that the system loads the recovery environment successfully.

Importance of an Emergency Boot Disk
• Helps repair corrupted system files
• Used to fix startup issues
• Allows virus or malware removal on non-booting systems
• Enables system restore and data recovery

# 2. How to Resolve Common Hardware Conflicts

Hardware conflicts occur when two or more devices attempt to use the same system resources such as IRQs, I/O addresses, DMA channels, or memory ranges. These conflicts may lead to system instability or device malfunction. The following methods can be used to resolve hardware conflicts:

(i) Use Device Manager to Identify Conflicts
Conflicted devices display a warning sign in Device Manager. Viewing device properties helps identify resource issues.

(ii) Update or Reinstall Device Drivers
Update drivers to the latest versions, roll back to previous versions, or reinstall if corrupted or incompatible.

(iii) Change Resource Settings Manually
Advanced users may change IRQ settings, I/O port addresses, and DMA channels via Device Manager to eliminate conflicts.

(iv) Disable or Remove Conflicting Devices
Disable unused devices in BIOS/UEFI or Device Manager to free system resources.

(v) Use BIOS/UEFI Settings
Resetting BIOS to default or updating firmware may fix compatibility and resource conflicts.

(vi) Ensure Hardware Compatibility
Verify that hardware components are compatible with the motherboard and operating system before installation.

# 3. Diagnosing Motherboard, RAM and CPU Issues

Effective diagnosis of computer hardware problems involves identifying symptoms, performing systematic tests, and verifying component functionality.

(i) Diagnosing Motherboard Issues
• Check physical symptoms such as failure to power, system freezes, or hardware not detected.
• Confirm power supply and connections are functional.
• Listen for POST beep codes to identify the failure area.
• Isolate components by booting with only CPU, RAM, and PSU.
• Inspect for physical damage like bulging capacitors or burnt circuits.

(ii) Diagnosing RAM Issues
• Typical symptoms: system crashes, BSOD errors, freezing, or app failures.
• Reseat and test RAM sticks individually.
• Run Windows Memory Diagnostic or MemTest86.
• Check RAM compatibility (speed, type) with the motherboard.

(iii) Diagnosing CPU Issues
• Symptoms include overheating, failure to boot, freezing, or poor performance.
• Check cooling fan, heat sink, and apply thermal paste if needed.
• Monitor CPU temperature via BIOS or system tools.
• Boot with minimal hardware to rule out other components.
• Inspect for bent pins or burn damage.

# 4. Data Backup and Recovery Methods

Backups protect data from loss caused by system failure, corruption, accidental deletion, or malware attacks.

Methods of Backing Up Data:
(i) External Storage Backup – Using USB or external drives.
(ii) Cloud Backup – Using services like Google Drive or OneDrive.
(iii) System Image Backup – Captures OS, settings, and files for full restoration.
(iv) NAS Backup – For office networks and multiple-user backups.
(v) Backup Software – Automated scheduled backups with encryption.

Data Recovery Methods:
(i) Restore from Backup Files – Copy back files from external or cloud storage.
(ii) Use System Restore or Recovery Tools – Repair settings without deleting data.
(iii) Restore System Image – Full OS and data restoration.
(iv) Use Data Recovery Software – E.g., Recuva or Disk Drill for undeleting files.
(v) Professional Data Recovery Services – For severely damaged storage devices.

Conclusion
A good backup strategy ensures quick data restoration and minimal loss, maintaining continuity after system failure.